CJ ENM 커머스부문 스튜디오 렌즈 구매 규격서

(Commodity Description)

| NO | Description | Q’TY |
| --- | --- | --- |
| 1 | LENS, ZOOM(STD) | 6 Set |
|  | A. Feature & Specifications - Application: 2/3” CMOS 4K Camera - Built-in Extender: 2x - Zoom Ratio / Iris: ≥27x / Auto and Manual - Max. focal length(1x): ≥180mm - Max. focal length(2x): ≥360mm - Max. aperture ratio: less than F1.5 - Angular field of view(1x): ≥72.8˚ - Angular field of view(2x): ≥40.5˚ - M.O.D from lens front: ≤0.6m - Remote control function for zoom, focus & return video. B. Compositions(total) - Zoom Lens: 6 set - Full-servo zoom & focus controller kit with clamp unit: 6 set - More than 3 years warranty - All necessary accessories for the complete function - Instruction & maintenance manual in English C. Nominated Model - Maker / Model: Fujifilm / UA27X6.5BESM - Maker / Model: Canon / UJ27X6.5BISS |  |
| 2 | LENS, ZOOM(EFP) | 6 Set |
|  | A. Feature & Specifications - Camera Sensor: UHD 2/3” CMOS - Built-in Extender: 2x - Zoom Ratio / Iris: ≥24x / Auto and Manual - Max. focal length(1x): ≥187mm - Max. focal length(2x): ≥374mm - Max. aperture ratio: less than F1.8 - Angular field of view(1x): ≥63.2˚ - Angular field of view(2x): ≥34.2˚ - M.O.D from lens front: ≤0.8m - Remote control function for zoom, focus & return video. - Macro available B. Compositions(total) - Zoom Lens with protection or UV filter and cover: 6 set - Full-servo zoom & focus controller kit with clamp unit: 6 set - More than 3 years warranty - All necessary accessories for the complete function - Instruction & maintenance manual in English C. Nominated Model - Maker / Model: Fujifilm / UA24X7.8BERD - Maker / Model: Canon / CJ25eX7.6B IASE S |  |
| 3 | LENS, WIDE(EFP) | 8 Set |
|  | A. Feature & Specifications - Camera Sensor: UHD 2/3” CMOS - Built-in Extender: 2x - Zoom Ratio / Iris: ≥14x / Auto and Manual - Max. focal length(1x): ≥63mm, ≥4.5mm - Max. focal length(2x): ≥126mm, ≥9mm - Max. aperture ratio: less than F1.8 - Angular field of view(1x): ≥93.6˚ - Angular field of view(2x): ≥56.1˚ - M.O.D from lens front: ≤0.3m - Remote control function for zoom, focus & return video. - Macro available B. Compositions(total) - Zoom Lens with protection or UV filter and cover: 6 set - Full-servo zoom & focus controller kit with clamp unit? - More than 3 years warranty - All necessary accessories for the complete function - Instruction & maintenance manual in English C. Nominated Model - Maker / Model: Fujifilm / UA14X4.5BERD - Maker / Model: Canon / CJ14eX4.3B IASE S |  |